Comments on the Various Factors on Placing Type

Necessarily  to be Considered

1.	This name has been positioned with respect to an approximately horizontal straight  	line – this is graphically undesirable! Wherever possible names should follow 		genuinely horizontal lines accurately, as the human eye is aware of even slight 		deviations. The use of a regular grid is recommended as a guide.

- The name is located with insufficient space being allowed between it and the related 	symbol. It is advisable that there should be the equivalent of at least half an upper- 		case character space between a name and a symbol.

- Placement has ignored the potential usefulness of the silhouette of the name 		because of the employment of lower case lettering.

- Reference between the symbol and its associated name is ambiguous.

- The ideal position of a name referring to a point symbol is on its right, but not on the same 	horizontal line.

- This type has not been positioned in the area to which the names pertain. Wherever 	possible names should be located within the territory to which they refer, or on the 		appropriate side of a river or boundary line.

- These names have been placed directly on top of the line to which they relate. This interrupts 	the letters and makes them difficult to read. Sufficient space, at least half of the size of a 	lower-case letter, should be allowed between the bottom of a name and its associated 	line symbol.

- Names and numbers must not be positioned on sharp curves, but rather where the shape 	of a feature is relatively smooth.

- Words  incorporating  characters which  reach below the normal base line (for example g, 	p, q and j) should be positioned where bends in a line symbol make their placement 		appropriate or alternatively be placed below the line. This is especially important in the 	case of straight lines.

- In areas which are congested with quantities of names and symbols, particular care must 	be taken to ensure unambiguous identification. This is   best   accomplished   by 		arranging   names around the  highly  detailed  area  rather than attempting to 		incorporate them in it.

- Names in this part of the map have not been adequately centred, and conse- 		quently do not extend over the full area to which they should refer. Whenever 		possible such names should stretch over approximately two thirds of the ter
	ritory to which they relate, and serve as a central axis for the region.

- The use of curved type is unnecessary in these areas. Names referring to areas 		should normally be placed horizontally in order to simplify their reading.

- The spacing  of the letters comprising these names does not relate naturally to the 	size of the individual characters. It is essential that the letters forming a name or title can be 	read and appreciated as a whole. If necessary the size of the type must be increased 	to enable this to occur.

- These names have not been positioned with respect to a regularly curved base line. 	The user's eye will experience difficulties in attempting to reassemble the complete words. 	Curved lettering, whether or not it is spaced, should always be arranged along a regular 	base line. This can be experimentally sketched onto the manuscript before the 		characters are positioned.

- The  intersection  of names at narrow angles always results in confusion for the 		reader. When, of necessity, two names have to cross they should be at as near to a 	right angle as possible.

- Type relating to an area, and having a straight base  line, should  never be positioned at 	an oblique angle. Horizonal placement, orthe curving of lettering, is always preferable.

- These names have been unnecessarily hyphenated and displayed on separate lines. 	In consequence it becomes difficult to read the name as a whole.

- Wherever possible names should read from the middle of the bottom edge of a map, 	and not be oriented towards the other edges.