## Construction methods for dasymetric maps

Imhof (1972, p. 168f) describes two methods for creating dasymetric maps.

### Method 1

On the basis of a dot map areas with approximate equal object densities are built. These areas are located according to different subjective aspects and vary depending on the cartographer. On this account different maps can result on the same base data. To improve this method, it is recommended to involve the given spatial cultural structures in the map design. To create areas with approximate equal population density, per example, the cartographer can take the living areas and their properties in account. Furthermore, the use of appropriate topographic detail maps is recommended. 
In case to small areas were generated, they are scaled up or aggregated with adjoining areas to keep the map readable. This causes inhomogenity within the new areas and the map's statement can be changed. But normally, the effect is minimal because of the classification. This method is appropriate for small map scales.

The following example shows a dot map and two possible results of a dasymetric map.

### Method 2

The second method presented by Imhof uses a given choropleth map with political structures as basis information. The administrative boundaries should be on a very low level such as municipality or quarter. This map is transformed into a smaller scaled map by generalisation. This method is quite simple but less accurate than the first method. This method is only appropriate for small map scales.

Besides the two construction methods of Imhof, so-called objective methods exist to detect the boundaries of areas of constant object density. Although, the methods are not really objective, as the subjectivity of the choice of the. Both presented methods use a dot map as construction basis.

### Voronoi method

Between neighbouring points, the vertical centre lines are drawn and joined. Thus, around each point results an area. Each point in this area is closer to the dot corresponding to it than to any other point of the map. With this area and the dot value the object density in this region is calculated. The calculated densities can then be sorted, classified and visualized in the map. 
In peripheral areas uncertainties can appear, as area boundaries are not defined exactly there. 

With the interactive tool VoroGlide you can find out graphically how the Voronoi method works. (http://www.pi6.fernuni-hagen.de/GeomLab/VoroGlide/)

The following example shows a dot map and the deduced dasymetric map.

### Distance method

The first step is the definition of marginal distances between two dots. Then, each point is connected with its nearest neighbouring points. With the distance among the points and the marginal distances, the dots are merged into density ranges and the corresponding areas are built. As the connections of dots do not make sense for every topic, this method is not appropriate for each map.