Other Scales

Some maps can

have a different west-east scale than

north-south scale. The Mercator map

( figure 1) which has a progressive scale

starting at the equator has to be pointed

out. The Mercator projection is used for

nautical charts because the course-lines

are presented as straight lines. All parallels

are stretched and the pole is not visible any

more as it lays in the infinite. See figure 2

for the special scale needed the Mercator

map.

# Figure 1: Mercatorian map (Brandenberger, 1996)

# Figure 2: Progressive scale (Imhof, 1950)

On some maps you might still find a slope diagram (figure 3) from which the angle of slope can be deducted. For the horizontal contour line interval, the inclination can either be extracted in:

# Figure 3: Slope Diagram – taken from a German topographic map at the scale of 1: 50 000 published in 1963 (Landesvermessungsamt Baden Württemberg 1963)

- Degree

- Hundredths / Percent

- Ratio

The description below figure 3 shows how it works:

- Measure the distance between point A and B which characterises the horizontal distance of the contour lines. Look at the       to find the solution of 10.0 meter.

- Its angle of slope in degree you find at this           position. Solution: 1° 18‘‘

- The angle of slope in percent at this           position: Solution: 2,3%

- And its expression in ratio:          Solution: 1: 44

If you have a map without any scale at all, apply the following procedure to find out the missing scale:

- Chose two significant points on a map where you can find a linear scale.

- Take the denominator of the scale (x1)

- Measure this distance in cm (y1) .

- Take the map without scale and find the same two significant points.

- Measure this distance in cm (y2)

- Calculate the scale of your map as follows:

y1	x2	y1

=     			x2 =  x1 *

y2	x1	y2

- x2 is the scale value. To get the map scale just built the reciprocal value of it:   1 : x2

Bibliographie:

Brandenberger, Ch., 1996. Verschiedene Aspekte und Projektionen für Weltkarten. Institut für Kartographie, ETH Zürich.

Imhof, E., 1950. Gelände und Karte. Eugen Rentsch Verlag. Erlenbach – Zürich

Landesvermessungsamt Baden Württemberg, 1963. Topographische Karte 1:50'000.