Act: Slope Diagram

Take this map excerpt at a scale of 1: 25 000 and the slope diagram ( 1:1 ) as working sheet:

Solution for A/ B:

- difference of elevation:  	50 m

- angle of slope in: degree :	~ 10°

- angle of slope in: percent:	~ 17,8 %

- angle of slope in: ratio:		~ 1: 5,8

Solution for C/ D:

- difference of elevation:		40 m

- angle of slope in: degree:	~ 2° 46‘

- angle of slope in: percent:	~ 4,8 %

- angle of slope in: ratio:		~ 1: 20

Determine the scale for the following map excerpt that is unknown:

Solution for scale:  ~ 1 : 39344 	( For a practical map scale you would like to use for mapping the nearest logical value lies at 1 : 40 000 –

the map would need to be re - scaled )